Lecture 1 – Basic Networking Concepts

1. Introduction to Computer Networks

Definition of a Network

A computer network is a collection of interconnected devices (computers, routers, servers, etc.) that communicate with each other using communication links.

Key Characteristics

- Devices exchange data.

- Communication occurs via wired or wireless media.

- Governed by protocols.

2. The Internet

Definition

The Internet is a global network of interconnected networks that communicate using standardized protocols.

Key Components

- Hosts (End Systems)

  - Computers, smartphones, IoT devices

  - Run applications

  - Located at the network edge

- Communication Links

  - Connect devices together

  - Examples:

    - Fiber optic

    - Copper cables

    - Wireless

    - Satellite

- Packet Switches

  - Forward data packets

  - Examples:

    - Routers

    - Switches

- Internet Service Providers (ISPs)

  - Organizations that provide Internet connectivity.

Internet Structure

The Internet consists of:

Network Edge

Where end devices (hosts) are located.

Examples:

- PCs

- Mobile phones

- Servers

Servers are often located in data centers.

Network Core

The central part of the network that connects edge devices.

Consists of:

- Routers

- High-speed links

Responsible for:

- Routing

- Forwarding packets

3. Internet as Infrastructure and Service

Internet as Infrastructure

Provides services for applications such as:

- Web browsing

- Email

- Video streaming

- Online gaming

- Social media

- Cloud computing

Internet as a Service Platform

Provides an API-like interface for applications.

Applications use the Internet through:

- Transport services

- Protocols

Example services:

- HTTP

- TCP

- IP

- WiFi

4. Protocols

Definition

A protocol is a set of rules that governs communication between devices.

Protocols define:

- Message format

- Order of messages

- Actions taken when messages are received

Example (Human Protocol)

Conversation:

- Person A: "What time is it?"

- Person B: "2:00"

Rules:

- Greeting

- Question

- Response

Example (Network Protocol)

Example communication:

- Client sends TCP connection request

- Server responds

- Client sends HTTP request

Examples of Internet Protocols

- HTTP – Web communication

- TCP – Reliable transport

- IP – Addressing and routing

- WiFi – Wireless communication

- Ethernet – Wired LAN communication

5. Data Communication

Definition

Data communication is the exchange of data between devices using a transmission medium.

Effectiveness of Data Communication

Three main factors:

1. Delivery

Data must reach the correct destination.

2. Accuracy

Data must arrive without errors.

3. Timeliness

Data must arrive within acceptable time.

6. Components of a Data Communication System

A communication system includes:

- Sender

  - Device that sends data.

- Receiver

  - Device that receives data.

- Message

  - Information being transmitted.

- Medium

  - Transmission path.

- Protocol

  - Rules governing communication.

7. Direction of Data Flow

Simplex

- One-direction communication only.

Example:

- TV broadcasting

Half-Duplex

- Both directions but not simultaneously.

Example:

- Walkie-talkies

Full-Duplex

- Both directions simultaneously.

Example:

- Telephone calls

8. Access Networks

Access networks connect end systems to the Internet.

Types of Access Networks

1. Residential Access Networks

Used in homes.

Examples:

- Cable Internet

- DSL

2. Institutional Access Networks

Used in organizations such as:

- Universities

- Companies

Technologies:

- Ethernet

- WiFi

3. Mobile Access Networks

Used by mobile devices.

Examples:

- 4G

- 5G

- WiFi

9. Cable Internet (HFC)

Hybrid Fiber Coaxial (HFC) network.

Characteristics

- Fiber connects ISP to neighborhoods

- Coaxial cable connects homes

Features

- Shared bandwidth

- Uses Frequency Division Multiplexing (FDM)

Typical speeds:

- Downstream: up to 1.2 Gbps

- Upstream: 30–100 Mbps

10. DSL (Digital Subscriber Line)

Uses existing telephone lines.

Components

- DSL modem

- DSLAM (Digital Subscriber Line Access Multiplexer)

Characteristics

- Dedicated connection to ISP

- Data and voice transmitted at different frequencies.

Typical speeds:

- Downstream: 24–52 Mbps

- Upstream: 3.5–16 Mbps

11. Home Networks

Typical home network components:

- Modem (DSL or Cable)

- Router

- Firewall

- WiFi access point

- Ethernet connections

Common speeds:

- Ethernet: 1 Gbps

- WiFi: 54–450 Mbps

12. Physical Media

Physical media carry bits across the network.

Guided Media

Signals travel through physical cables.

Examples:

Twisted Pair

- Two copper wires twisted together

- Used in Ethernet

Speeds:

- Up to 10 Gbps

Coaxial Cable

- Two concentric copper conductors

- Used in cable networks

Supports multiple channels.

Fiber Optic Cable

- Uses light signals

- Very high speed

Advantages:

- High bandwidth

- Low error rate

- Immune to electromagnetic interference

Speed:

- 10–100+ Gbps

Unguided Media

Signals travel through air.

Examples:

- WiFi

- Cellular networks

- Bluetooth

- Satellite communication

13. Packet Switching

Data is divided into packets before transmission.

Packets travel independently through the network.

Routers forward packets to their destination.

Store-and-Forward Transmission

Routers must receive the entire packet before forwarding it.

Transmission delay formula:

dtrans=LR

Where:

- L = packet length (bits)

- R = transmission rate (bits/sec)

14. Queueing

Packets may wait in a queue when the router is busy.

Occurs when:

Arrival rate > transmission rate.

Results:

- Delay

- Packet loss (if buffer full)

15. Circuit Switching

Used in traditional telephone networks.

Characteristics:

- Dedicated communication path

- Reserved bandwidth

- No resource sharing

Examples:

- Telephone calls

Multiplexing Methods

Frequency Division Multiplexing (FDM)

- Different signals transmitted using different frequencies.

Time Division Multiplexing (TDM)

- Time divided into slots for each user.

16. Packet vs Circuit Switching

Packet Switching

Advantages:

- Efficient resource sharing

- Suitable for bursty data

Disadvantages:

- Possible delay

- Possible packet loss

Circuit Switching

Advantages:

- Guaranteed bandwidth

- Predictable performance

Disadvantages:

- Inefficient resource use.

17. Internet Structure

The Internet consists of multiple layers of ISPs.

Types of ISPs

- Access ISPs

- Regional ISPs

- Tier-1 ISPs

Internet Exchange Points (IXP)

Facilities where ISPs connect to exchange traffic.

Content Provider Networks

Large companies operate their own networks.

Examples:

- Google

- Facebook

- Microsoft

Purpose:

- Deliver content faster to users.

18. Network Performance

Important metrics:

- Delay

- Packet loss

- Throughput

19. Types of Packet Delay

Total nodal delay:

dnodal=dproc+dqueue+dtrans+dprop

1. Processing Delay

Time to process packet header.

2. Queueing Delay

Time waiting in router queue.

Depends on congestion.

3. Transmission Delay

dtrans=LR

4. Propagation Delay

dprop=ds

Where:

- d = distance

- s = propagation speed

20. Packet Loss

Occurs when router buffers become full.

Packet is dropped.

Recovery:

- Retransmission

21. Throughput

Throughput = rate at which data is successfully delivered

Measured in:

- bits per second (bps)

Bottleneck Link

The slowest link determines the end-to-end throughput.

22. Network Security

The Internet was not originally designed with strong security.

Common threats include:

Packet Sniffing

Attackers capture packets.

Tools:

- Wireshark

IP Spoofing

Attacker sends packets with a fake source address.

Denial of Service (DoS)

Attackers overwhelm a server with traffic.

Goal:

- Make services unavailable.

23. Security Defenses

Authentication

Verify identity.

Confidentiality

Encryption protects data.

Integrity

Ensures data is not modified.

Access Control

Restricts network access.

Examples:

- Passwords

- VPN

Firewalls

Network security systems that filter traffic.

24. Protocol Layering

To manage complexity, networks use layers.

Each layer performs specific functions.

25. Internet Protocol Stack

The Internet uses a 5-layer architecture.

1. Application Layer

Supports applications.

Examples:

- HTTP

- DNS

- SMTP

2. Transport Layer

Process-to-process communication.

Protocols:

- TCP

- UDP

3. Network Layer

Routing packets.

Protocol:

- IP

4. Link Layer

Communication between neighboring nodes.

Examples:

- Ethernet

- WiFi

5. Physical Layer

Transmission of bits.

Examples:

- Electrical signals

- Optical signals

26. Encapsulation

Each layer adds its own header.

Data units:

Application → Message
Transport → Segment
Network → Datagram
Link → Frame

27. OSI Reference Model

7-layer model:

- Application

- Presentation

- Session

- Transport

- Network

- Data Link

- Physical

Difference from Internet Model

Internet stack does not include:

- Presentation layer

- Session layer

Their functions are handled in the application layer.